Sunday Minibus Check Sheet

NH56 FWP  ·  Ford Transit  ·  2007 · white · manual

Backup for the phone app. Complete before the first pick up. Allow ten minutes.

Before you touch the keys Only drivers on the church register may take this bus out. Items marked * stop the bus if you find a defect. The ON THIS BUS notes are its own fault history: read them.

## A. Outside the bus

Walk clockwise from the driver's door.

| Item | What to check | OK / Def |
| --- | --- | --- |
| Tyres * | Tread full width, edge wear, cuts or bulges, pressures. Include the spare. Look for flat spots. |  |
| Wheel nuts * | All present. Marker arrows lined up. Rust weeping from a nut means it has moved. |  |
| Lights | Sidelights, dipped, main, indicators, hazards, brake, reverse, fogs, plate lamp. Check the lenses too. ON THIS BUS: failed on a cracked rear stop lamp lens 2025. Check lenses, not just bulbs. |  |
| Front brakes * | Through the front wheels at the disc face. Heavy scoring, a deep lip, or one side much cleaner. |  |
| Body and glass | Chips or cracks in the wiper area. Sliding door runs freely on its track. |  |
| Rust and structure | Sills, arches, step mounts, and where the body meets the chassis. Flaking or soft metal, not staining. ON THIS BUS: cab mountings all four corners repaired and inspected April 2026. Surrounding metal is the same age: look hard at sills, underbody and chassis joints. |  |
| Underneath | Fresh oil, coolant or fuel on the road after standing all week. |  |
| Brake pipes and lines * | Along the chassis rails and behind each wheel: damp or wet patches, or a pipe flaking not just dusty. ON THIS BUS: a brake pipe leak was a dangerous defect in January 2025. Pipes of this age go one at a time. Any damp patch is a stop. |  |
| Suspension and ride height | Sitting level, not down at one corner. Knocking over the first bumps. ON THIS BUS: anti-roll bar ball joint replaced 2025, spring bush flagged 2015. |  |
| Exhaust | Secure. Nothing cut away underneath. Listen for a change in note at start up. |  |
| Number plates | Clean, readable and secure front and rear. |  |

## B. Under the bonnet

Engine cold, before the first start.

| Item | What to check | OK / Def |
| --- | --- | --- |
| Engine oil | Between min and max. ABOVE max is a warning sign, not a good thing. Report it. |  |
| Coolant | Between min and max on the tank, engine cold. |  |
| Brake fluid * | At or near max. If it has dropped, report it. Do not just top it up. ON THIS BUS: given the pipe history, a level that has dropped means a leak until proven otherwise. |  |
| Screenwash | Full. Winter mix from October onwards. |  |
| Belts and hoses | Look only. Cracking, fraying, or a hose swollen at the clip. |  |

## C. Inside the saloon

Where the passengers sit. Take your time here.

| Item | What to check | OK / Def |
| --- | --- | --- |
| Seat belts * | Pull each belt fully out, check for fraying, click in, check it locks and releases. Every seat. |  |
| Seat anchorages * | Push and pull each frame. Movement at the floor is a defect. Do not carry passengers in it. |  |
| Rear step and thresholds | Fixed rear step secure, not loose or lifting, not slippery. Side door threshold free of a trip lip. No retractable step, so check the drop is clear and lit. |  |
| Emergency exits * | Rear door and side exit open from inside. Roof hatch works. Nothing stored in front of an exit. |  |
| Gangway and hammers | Gangway clear, floor dry. Break glass hammers clipped in, not loose. |  |
| Fire extinguisher | In date, gauge in the green, seal unbroken, bracket tight. |  |
| First aid kit | Present, sealed, contents in date. |  |
| General condition | No sharp edges, exposed metal or loose wiring. |  |

## D. In the cab

Engine running, before anyone gets on.

| Item | What to check | OK / Def |
| --- | --- | --- |
| ABS warning lamp * | On at ignition then out. If it stays on, the bus does not run. |  |
| Other warning lights | All lamps on then out. Amber engine or DPF light means report before loading. |  |
| Brakes * | Firm pedal, no sinking. Handbrake holds on a slope. Controlled stop at low speed before the first pick up. Listen for grinding or pulling. ON THIS BUS: parking brake and cable put right early 2025. Test the handbrake on a slope every time. |  |
| Steering * | No excess free play. No knocking on full lock. |  |
| Clutch and gears | Bite point where you expect it, not at the top. All gears select cleanly including reverse. No slipping. |  |
| Mirrors and wipers | Mirrors clean, adjusted, secure. Blades not torn or smearing. |  |
| Horn, heater, demist | All working. Insurance certificate in date and on board. |  |

## E. Sign here, every week

Complete this even when nothing is wrong. Writing NONE is the record that the check was done.

| Date | Driver name | Role | Mileage | Signature |
| --- | --- | --- | --- | --- |

| Defects found (write NONE if the bus is clear) | Safe to drive? Y / N |
| --- | --- |

The bus does not run if the defect involves any of these: Brakes ¦ Steering ¦ Tyres ¦ Wheel nuts ¦ Brake pipes ¦ Brake fluid ¦ Seat belts ¦ Seat anchorages ¦ ABS lamp ¦ Any emergency exit Ring Asim Bassey on 07377 634214 and arrange lifts. Nobody will be upset with you for stopping the bus. Hand this sheet in after the return journey.