Sunday Minibus Check Sheet

YS70 PWE  ·  Ford Transit 460 Trend  ·  2020 · silver · manual

Backup for the phone app. Complete before the first pick up. Allow ten minutes.

Before you touch the keys Only drivers on the church register may take this bus out. Items marked * stop the bus if you find a defect. The ON THIS BUS notes are its own fault history: read them.

## A. Outside the bus

Walk clockwise from the driver's door.

| Item | What to check | OK / Def |
| --- | --- | --- |
| Tyres * | Tread full width, edge wear, cuts or bulges, pressures. Include the spare. Look for flat spots. ON THIS BUS: worn on the edge at past tests, so report edge wear even with plenty of tread. |  |
| Wheel nuts * | All present. Marker arrows lined up. Rust weeping from a nut means it has moved. |  |
| Lights | Sidelights, dipped, main, indicators, hazards, brake, reverse, fogs, plate lamp. Check the lenses too. |  |
| Front brakes * | Through the front wheels at the disc face. Heavy scoring, a deep lip, or one side much cleaner. ON THIS BUS: pads were dangerously thin in 2024 and worn again within 1,360 miles. |  |
| Body and glass | Chips or cracks in the wiper area. Sliding door runs freely on its track. |  |
| Rust and structure | Sills, arches, step mounts, and where the body meets the chassis. Flaking or soft metal, not staining. |  |
| Underneath | Fresh oil, coolant or fuel on the road after standing all week. |  |
| Brake pipes and lines * | Along the chassis rails and behind each wheel: damp or wet patches, or a pipe flaking not just dusty. |  |
| Suspension and ride height | Sitting level, not down at one corner. Knocking over the first bumps. |  |
| Exhaust | Secure. Nothing cut away underneath. Listen for a change in note at start up. |  |
| Number plates | Clean, readable and secure front and rear. |  |

## B. Under the bonnet

Engine cold, before the first start.

| Item | What to check | OK / Def |
| --- | --- | --- |
| Engine oil | Between min and max. ABOVE max is a warning sign, not a good thing. Report it. |  |
| Coolant | Between min and max on the tank, engine cold. |  |
| Brake fluid * | At or near max. If it has dropped, report it. Do not just top it up. ON THIS BUS: the front brakes already have history, so a drop means a leak until proven otherwise. |  |
| Screenwash | Full. Winter mix from October onwards. |  |
| AdBlue | Gauge or dash message. Never run empty or the engine will not restart. |  |
| Belts and hoses | Look only. Cracking, fraying, or a hose swollen at the clip. |  |

## C. Inside the saloon

Where the passengers sit. Take your time here.

| Item | What to check | OK / Def |
| --- | --- | --- |
| Seat belts * | Pull each belt fully out, check for fraying, click in, check it locks and releases. Every seat. |  |
| Seat anchorages * | Push and pull each frame. Movement at the floor is a defect. Do not carry passengers in it. |  |
| Retractable step | Deploys and retracts fully and smoothly. Tread not worn smooth. Step light works. ON THIS BUS: flagged at three tests since 2023, never repaired. The weakest thing on this bus. |  |
| Emergency exits * | Rear door and side exit open from inside. Roof hatch works. Nothing stored in front of an exit. |  |
| Gangway and hammers | Gangway clear, floor dry. Break glass hammers clipped in, not loose. |  |
| Fire extinguisher | In date, gauge in the green, seal unbroken, bracket tight. |  |
| First aid kit | Present, sealed, contents in date. |  |
| General condition | No sharp edges, exposed metal or loose wiring. |  |

## D. In the cab

Engine running, before anyone gets on.

| Item | What to check | OK / Def |
| --- | --- | --- |
| ABS warning lamp * | On at ignition then out. If it stays on, the bus does not run. ON THIS BUS: failed its MOT on this lamp in June 2026 and passed the same day. |  |
| Other warning lights | All lamps on then out. Amber engine or DPF light means report before loading. |  |
| Brakes * | Firm pedal, no sinking. Handbrake holds on a slope. Controlled stop at low speed before the first pick up. Listen for grinding or pulling. ON THIS BUS: the front brake wear rate does not add up. Grinding or pulling is a stop, not a note. |  |
| Steering * | No excess free play. No knocking on full lock. |  |
| Clutch and gears | Bite point where you expect it, not at the top. All gears select cleanly including reverse. No slipping. |  |
| Mirrors and wipers | Mirrors clean, adjusted, secure. Blades not torn or smearing. |  |
| Horn, heater, demist | All working. Insurance certificate in date and on board. |  |

## E. Sign here, every week

Complete this even when nothing is wrong. Writing NONE is the record that the check was done.

| Date | Driver name | Role | Mileage | Signature |
| --- | --- | --- | --- | --- |

| Defects found (write NONE if the bus is clear) | Safe to drive? Y / N |
| --- | --- |

The bus does not run if the defect involves any of these: Brakes ¦ Steering ¦ Tyres ¦ Wheel nuts ¦ Brake pipes ¦ Brake fluid ¦ Seat belts ¦ Seat anchorages ¦ ABS lamp ¦ Any emergency exit Ring Asim Bassey on 07377 634214 and arrange lifts. Nobody will be upset with you for stopping the bus. Hand this sheet in after the return journey.